### Chapter 1: Computer Networks and the Internet

- In a store-and-forward packet switching network, which of the following is TRUE regarding the transmission time of a packet from a source to a destination over multiple hops?
a) Transmission time depends solely on the number of hops.
b) Each switch must receive the entire packet before forwarding it to the next switch.
c) Transmission time decreases as the packet size increases.
d) Packets are forwarded as soon as they begin arriving at the switch.

Answer: b) Each switch must receive the entire packet before forwarding it to the next switch.

- Suppose a packet of 10,000 bits travels over three links. The transmission rates of the links are 100 Mbps, 50 Mbps, and 25 Mbps, respectively. What is the bottleneck link in this network?
a) 100 Mbps
b) 50 Mbps
c) 25 Mbps
d) None of the above

Answer: c) 25 Mbps

- Which of the following is a fundamental difference between a circuit-switched and a packet-switched network?
a) Circuit-switched networks reserve resources end-to-end for the duration of the connection.
b) Packet-switched networks require a pre-established path before data transmission begins.
c) Circuit-switched networks provide only best-effort delivery.
d) Packet-switched networks cannot handle real-time applications effectively.

Answer: a) Circuit-switched networks reserve resources end-to-end for the duration of the connection.

- In the context of layered network architecture, which of the following correctly describes the principle of "encapsulation"?
a) Only the physical layer is allowed to modify data.
b) Lower layers process only their own headers, discarding higher layer headers.
c) Each layer adds its own header information to the data from the layer above.
d) Data flows from the transport layer directly to the network layer, bypassing intermediate layers.

Answer: c) Each layer adds its own header information to the data from the layer above.

- How is the propagation delay in a network link calculated?
a) Propagation delay = Distance / Transmission rate
b) Propagation delay = Packet size / Link capacity
c) Propagation delay = Distance / Propagation speed
d) Propagation delay = Packet size / Propagation speed

Answer: c) Propagation delay = Distance / Propagation speed

- If a network’s transmission delay is significantly less than its propagation delay, what does this imply about the network?
a) The link has a high bandwidth.
b) The link is very long in distance.
c) The link has a low transmission rate.
d) The packet size is very large.

Answer: b) The link is very long in distance.

- Which of the following best defines the "bandwidth-delay product" in networking?
a) It measures the total transmission delay of a packet.
b) It represents the amount of data that can be "in flight" on the network link at any given time.
c) It quantifies the error-correction capability of a network link.
d) It determines the maximum possible packet size.

Answer: b) It represents the amount of data that can be "in flight" on the network link at any given time.

- What is the primary reason for using the OSI reference model in networking?
a) To enforce rigid protocol standards across all network types
b) To allow for standardized communication protocols between diverse systems
c) To improve data transmission speed
d) To simplify the process of IP addressing

Answer: b) To allow for standardized communication protocols between diverse systems

- What is the effect of doubling the transmission rate (in bits per second) of a link on the total end-to-end delay for a large file being transmitted?
a) The total delay is halved.
b) The total delay remains unchanged.
c) The total delay increases by a factor of two.
d) The total delay reduces, but not necessarily by half.

Answer: d) The total delay reduces, but not necessarily by half.

- How does a "Tier-1 ISP" differ from lower-tier ISPs?
a) Tier-1 ISPs are directly connected to the end-user.
b) Tier-1 ISPs do not need to pay for peering agreements with other ISPs.
c) Tier-1 ISPs only operate within national boundaries.
d) Tier-1 ISPs only handle local traffic and not global data.

Answer: b) Tier-1 ISPs do not need to pay for peering agreements with other ISPs.

### Chapter 2: Application Layer

- Which of the following accurately describes the role of the Transport Layer in a client-server application such as HTTP?
a) It is responsible for setting up a persistent connection between client and server.
b) It manages session cookies between the client and server.
c) It provides reliable data transfer between the client and server.
d) It forwards HTTP requests directly to the server’s application layer.

Answer: c) It provides reliable data transfer between the client and server.

- In a distributed system using peer-to-peer architecture, what is the primary reason for data replication across multiple peers?
a) To enhance security
b) To increase availability and fault tolerance
c) To reduce the load on a central server
d) To improve the speed of data retrieval

Answer: b) To increase availability and fault tolerance

- Considering a file of size 5MB is being transferred using FTP in a reliable transport protocol, how would the performance be affected if the connection has a high round-trip time (RTT)?
a) It would increase transfer speed due to the pipeline nature of TCP.
b) It would increase total transfer time due to acknowledgment delays for each segment.
c) It would have no impact because FTP does not depend on RTT.
d) It would decrease transfer time because of selective acknowledgments.

Answer: b) It would increase total transfer time due to acknowledgment delays for each segment.

- Which of the following describes the role of the “ephemeral port number” in a client-server connection?
a) It identifies the protocol used in the communication.
b) It serves as the return port number for client-to-server communication.
c) It allows multiple servers to run simultaneously on a single machine.
d) It is assigned temporarily to identify an ongoing client-side process.

Answer: d) It is assigned temporarily to identify an ongoing client-side process.

- When an HTTP client sends a "POST" request to a server, what is typically happening?
a) The client is requesting a resource from the server.
b) The client is uploading data to the server for processing.
c) The client is establishing a secure connection with the server.
d) The client is requesting data from another user via the server.

Answer: b) The client is uploading data to the server for processing.

- In DNS, what role does a recursive DNS resolver play in resolving domain names?
a) It holds authoritative records for a particular domain.
b) It queries other DNS servers on behalf of the client to resolve domain names.
c) It translates IP addresses back to domain names.
d) It stores cached DNS records for quick retrieval.

Answer: b) It queries other DNS servers on behalf of the client to resolve domain names.

- Which of the following would result in a violation of HTTP’s stateless nature?
a) Storing user authentication credentials on the server
b) Using the GET method to retrieve data
c) Making consecutive requests for different resources
d) Relying on HTTP headers to manage session states

Answer: d) Relying on HTTP headers to manage session states

- Which of the following optimizes content delivery in a Content Delivery Network (CDN)?
a) Using a centralized server to serve all global users
b) Caching content on multiple geographically distributed servers
c) Compressing all traffic between the user and the server
d) Encrypting data to speed up transmission

Answer: b) Caching content on multiple geographically distributed servers

- Which of the following correctly describes the difference between a "persistent" and "non-persistent" HTTP connection?
a) A persistent connection is maintained across multiple requests between the client and server.
b) A persistent connection closes after each request is completed.
c) A non-persistent connection allows pipelining of multiple requests without closing the connection.
d) Non-persistent connections improve performance by reducing latency.

Answer: a) A persistent connection is maintained across multiple requests between the client and server.

- In DNS, a CNAME record maps which of the following?
a) An IP address to a domain name
b) A domain alias to another canonical domain name
c) A mail server to its IP address
d) A service identifier to a domain name

Answer: b) A domain alias to another canonical domain name